PDP-11 Simulator Usage

19-Aug-2021

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2021, written by Robert M Supnik

Copyright (c) 1993-2021, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	4

2	PDP-11 Features	5

2.1	CPU and System	6

2.1.1	CPU	6

2.1.2	System Registers (SYSTEM)	9

2.2	I/O Devices	10

2.2.1	Unibus and Qbus DMA Devices	10

2.2.2	I/O Device Addressing	10

2.3	Programmed I/O Devices	11

2.3.1	PC11 Paper Tape Reader (PTR)	11

2.3.2	PC11 Paper Tape Punch (PTP)	12

2.3.3	DL11 Terminal Input (TTI)	12

2.3.4	DL11 Terminal Output (TTO)	13

2.3.5	LP11 Line Printer (LPT)	13

2.3.6	KW11-L Line-Time Clock (CLK)	14

2.3.7	KW11-P Programmable Clock (PCLK)	14

2.3.8	TA11/TA60 Cassette Tape (CT)	15

2.4	Floppy Disk Drives	15

2.4.1	RX11/RX01 Floppy Disk (RX)	15

2.4.2	RX211/RX02 Floppy Disk (RY)	16

2.5	Cartridge Disk Drives	17

2.5.1	RK11/RK05 Cartridge Disk (RK)	17

2.5.2	RK611/RK06,RK07 Cartridge Disk (HK)	18

2.5.3	RL11(RLV12)/RL01,RL02 Cartridge Disk (RL)	19

2.6	Massbus Subsystems	20

2.6.1	RH70/RH11 Massbus Adapters (RHA, RHB, RHC)	20

2.6.2	RP04/05/06/07, RM02/03/05/80 Disk Pack Drives (RP)	20

2.6.3	TM02/TM03/TE16/TU45/TU77 Magnetic Tapes (TU)	21

2.6.4	RS03/RS04 Fixed Head Disks	22

2.7	RQDX3/UDA50 MSCP Disk Controllers (RQ, RQB, RQC, RQD)	23

2.8	Fixed Head Disks	24

2.8.1	RC11 Fixed Head Disk (RC)	24

2.8.2	RF11/RS11 Fixed Head Disk (RF)	25

2.9	TC11/TU56 DECtape (DT)	26

2.10	Magnetic Tape Controllers	27

2.10.1	TM11 Magnetic Tape (TM)	27

2.10.2	TS11/TSV05 Magnetic Tape (TS)	28

2.10.3	TQK50 TMSCP Disk Controller (TQ)	29

2.11	Communications Devices	30

2.11.1	DC11 Additional Terminal Interfaces (DCI/DCO)	30

2.11.2	KL11/DL11 Additional Terminal Interfaces (DLI/DLO)	32

2.11.3	DZ11 Terminal Multiplexer (DZ)	33

2.11.4	DHQ11 Terminal Multiplexer (VH)	34

2.12	Ethernet Controllers	36

2.12.1	DELQA-T/DELQA/DEQNA Qbus Ethernet Controllers (XQ, XQB)	36

2.12.2	DELUA/DEUNA Unibus Ethernet Controllers (XU, XUB)	37

2.13	CR11/CD11 Card Reader (CR)	37

2.14	Arithmetic Options	39

2.14.1	KE11A Extended Arithmetic Option (KE)	39

2.14.2	KG11A Communications Arithmetic Option (KG)	40

3	Symbolic Display and Input	40

This memorandum documents the DEC PDP-11 simulator.

# Simulator Files

To compile the PDP-11, you must define VM_PDP11 as part of the compilation command line.  If you want expanded file support, you must also define USE_INT64 and USE_ADDR64 as part of the compilation command line.

sim/		scp.h

sim_console.h

sim_defs.h

sim_ether.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_ether.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/pdp11/	pdp11_cpumod.h

pdp11_cr_dat.h

pdp11_defs.h

pdp11_mscp.h

pdp11_uqssp.h

pdp11_xq.h

pdp11_xq_bootrom.h

pdp11_cpu.c

pdp11_cpumod.c

pdp11_cr.c

pdp11_dc.c

pdp11_dl.c

pdp11_dz.c

pdp11_fp.c

pdp11_hk.c

pdp11_ke.c

pdp11_kg.c

pdp11_io.c

pdp11_lp.c

pdp11_pclk.c

pdp11_pt.c

pdp11_rc.c

pdp11_rf.c

pdp11_rh.c

pdp11_rk.c

pdp11_rl.c

pdp11_rp.c

pdp11_rq.c

pdp11_rx.c

pdp11_ry.c

pdp11_stddev.c

pdp11_sys.c

pdp11_ta.c

pdp11_tc.c

pdp11_tm.c

pdp11_tq.c

pdp11_ts.c

pdp11_tu.c

pdp11_vh.c

pdp11_xq.c

pdp11_xu.c

# PDP-11 Features

The PDP-11 simulator is configured as follows:

device name(s)	simulates

CPU			PDP-11 CPU with 256KB of memory

PTR,PTP		PC11 paper tape reader/punch

TTI,TTO		DL11 console terminal

CR			CR11/CD11 card reader

LPT			LP11 line printer

CLK			KW11-L line frequency clock

PCLK			KW11-P programmable clock

DCI,DCO		DC11 additional serial lines (up to 16)

DLI,DLO		KL11/DL11 additional serial lines (up to 16)

DZ			DZ11 8-line terminal multiplexer (up to 4)

VH			DHU11/DHQ11 8-line terminal multiplexer (up to 4)

RK			RK11/RK05 cartridge disk controller with eight drives

HK			RK611/RK06,RK07 cartridge disk controller with eight

Drives

RC			RC11 fixed head disk

RF			RF11/RS11 fixed head disk

RL			RL11(RLV12)/RL01,RL02 cartridge disk controller with

four drives

RH			RH11/RH70 Massbus adapter (up to 2)

RP			RP04/05/06/07, RM02/03/05/80 Massbus disks with eight

drives

RQ			RQDX3/UDA50 MSCP controller with four drives

RQB			second RQDX3/UDA50 MSCP controller with four drives

RQC			third RQDX3/UDA50 MSCP controller with four drives

RQD			fourth RQDX3/UDA50 MSCP controller with four drives

RX			RX11/RX01 floppy disk controller with two drives

RY			RX211/RX01 floppy disk controller with two drives

TA			TA11/TU60 cassette controller with two drives

TC			TC11/TU56 DECtape controller with eight drives

TM			TM11/TU10 magnetic tape controller with eight drives

TS			TS11/TSV05 magnetic tape controller with one drive

TQ			TQK50/TU81 TMSCP magnetic tape controller with four

drives

TU			TM02/TM03 magnetic tape formatter with eight drives

XQ			DELQA/DEQNA Qbus Ethernet controller

XQB			second DELQA/DEQNA Qbus Ethernet controller

XU			DELUA/DEUNA Unibus Ethernet controller

XUB			Second DELUA/DEUNA Unibus Ethernet controller

KE			KE11A extended arithmetic option

KG			KG11A communications arithmetic option

The DZ, VH, DCI/DCO, DLI/DLO, RK, HK, RC, RF, RL, RP, RQ, RQB, RQC, RQD, RX, RY, TA, TC, TM, TS, TQ, XQ, XQB, XU, XUB, KE, and KG devices can be set DISABLED.  DCI/DCO, DLI/DLO, RC, RF, RQB, RQC, RQD, RY, TA, TS, VH, XQB, XU, XUB, KE, and KG are disabled by default.

The PDP-11 simulator implements several unique stop conditions:

- Abort during exception vector fetch, and register STOP_VEC is set

- Abort during exception stack push, and register STOP_SPA is set

- Trap condition 'n' occurs, and register STOP_TRAP<n> is set

- Wait state entered, and no I/O operations outstanding (i.e., no interrupt can ever occur)

- A simulated DECtape runs off the end of its reel, and flag STOP_OFFR is set

The LOAD command supports standard binary format tapes.  The DUMP command is not implemented.

## CPU and System

### CPU

The CPU options include CPU type, CPU instruction set options for the specified type, and the size of main memory.

SET CPU 11/03			set CPU type to 11/03

SET CPU 11/04			set CPU type to 11/04

SET CPU 11/05			set CPU type to 11/05

SET CPU 11/20			set CPU type to 11/20

SET CPU 11/23			set CPU type to 11/23

SET CPU 11/23+			set CPU type to 11/23+

SET CPU 11/24			set CPU type to 11/24

SET CPU 11/34			set CPU type to 11/34

SET CPU 11/40			set CPU type to 11/40

SET CPU 11/44			set CPU type to 11/44

SET CPU 11/45			set CPU type to 11/45

SET CPU 11/53			set CPU type to 11/53

SET CPU 11/60			set CPU type to 11/60

SET CPU 11/70			set CPU type to 11/70

SET CPU 11/73			set CPU type to 11/73

SET CPU 11/73B			set CPU type to 11/73B

SET CPU 11/83			set CPU type to 11/83

SET CPU 11/84			set CPU type to 11/84

set CPU 11/93			set CPU type to 11/93

set CPU 11/94			set CPU type to 11/94

SET CPU U18				deprecated; same as 11/45

SET CPU URH11			deprecated; same as 11/84

SET CPU URH70			deprecated; same as 11/70

SET CPU Q22				deprecated; same as 11/73

SET CPU NOEIS			disable EIS instructions

SET CPU EIS				enable EIS instructions

SET CPU NOFIS			disable FIS instructions

SET CPU FIS				enable FIS instructions

SET CPU NOFPP			disable FPP instructions

SET CPU FPP				enable FPP instructions

SET CPU NOCIS			disable CIS instructions

SET CPU CIS				enable CIS instructions

SET CPU NOBEVENT			disable BEVENT interrupt

SET CPU BEVENT			enable BEVENT interrupt

SET CPU 16K				set memory size = 16KB

SET CPU 32K				set memory size = 32KB

SET CPU 48K				set memory size = 48KB

SET CPU 64K				set memory size = 64KB

SET CPU 96K				set memory size = 96KB

SET CPU 128K			set memory size = 128KB

SET CPU 192K			set memory size = 192KB

SET CPU 256K			set memory size = 256KB

SET CPU 384K			set memory size = 384KB

SET CPU 512K			set memory size = 512KB

SET CPU 768K			set memory size = 768KB

SET CPU 1024K (or 1M)		set memory size = 1024KB

SET CPU 2048K (or 2M)		set memory size = 2048KB

SET CPU 3072K (or 3M)		set memory size = 3072KB

SET CPU 4096K (or 4M)		set memory size = 4096KB

The CPU types and their capabilities are shown in the following table:

type	bus	mem	MMU?	Umap?	EIS?	FIS?	FPP?	CIS?	BEVENT?

11/03	 Q	64K	no	no	std	opt	no	no	opt

11/04	 U	64K	no	no	no	no	no	no	no

11/05	 U	64K	no	no	no	no	no	no	no

11/20	 U	64K	no	no	no	no	no	no	no

11/23	 Q	4M	opt	no	std	no	opt	opt	opt

11/23+ Q	4M	std	no	std	no	opt	opt	no

11/24	 U	4M	std	std	std	no	opt	opt 	no

11/34	 U	256K	std	no	std	no	opt	no 	no

11/40	 U	256K	opt	no	std	opt	no	no 	no

11/44	 U	4M	std	std	std	no	opt	opt 	no

11/45	 U	256K	opt	no	std	no	opt	no 	no

11/53	 Q	4M	std	no	std	no	std	opt 	no

11/60	 U	256K	std	no	std	no	std	no 	no

11/70	 U	4M	std	std	std	no	opt	no 	no

11/73	 Q	4M	std	no	std	no	std	opt 	no

11/73B Q	4M	std	no	std	no	std	opt 	no

11/83	 Q	4M	std	no	std	no	std	opt 	no

11/84	 U	4M	std	std	std	no	std	opt 	no

11/93	 Q	4M	std	no	std	no	std	opt 	no

11/94	 U	4M	std	std	std	no	std	opt 	no

If a capability is standard, it cannot be disabled; if a capability is not included, it cannot be enabled.

The CPU implements a SHOW command to display the I/O address assignments:

SHOW CPU IOSPACE			show I/O space address assignments

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 256KB.  If memory size is increased to more than 256KB, or the bus structure is changed, the simulator disables peripherals that can't run in the current bus structure.

These switches are recognized when examining or depositing in CPU memory:

-v					interpret address as virtual

-t					if mem mgt enabled, force data space

-k					if mem mgt enabled, force kernel mode

-s					if mem mgt enabled, force supervisor mode

-u					if mem mgt enabled, force user mode

-p					if mem mgt enabled, force previous mode

CPU registers include the architectural state of the PDP-11 processor as well as the control registers for the interrupt system.

name			size		comments

PC			16		program counter

R0..R5		16		R0..R5, current register set

SP			16		stack pointer, current mode

R00..R05		16		R0..R5, register set 0

R10..R15		16		R0..R5, register set 1

KSP			16		kernel stack pointer

SSP			16		supervisor stack pointer

USP			16		user stack pointer

PSW			16		processor status word

CM			2	  	current mode, PSW<15:14>

PM			2	  	previous mode, PSW<13:12>

RS			2	  	register set, PSW<11>

IPL			3	  	interrupt priority level, PSW<7:5>

T			1	  	trace bit, PSW<4>

N			1	  	negative flag, PSW<3>

Z			1	  	zero flag, PSW<2>

V			1	  	overflow flag, PSW<1>

C			1	  	carry flag, PSW<0>

PIRQ			16		programmed interrupt requests

STKLIM		16		stack limit

FAC0H..FAC5H	32		FAC0..FAC5, high 32 bits

FAC0L..FAC5L	32		FAC0..FAC5, low 32 bits

FPS			16		floating point status

FEA			16		floating exception address

FEC			4		floating exception code

MMR0 to 3		16		memory management registers 0 to 3

{K/S/U}{I/D}{PAR/PDR}{0..7}

16		memory management registers

IREQ[0:7]		32		interrupt pending flags, IPL 0 to 7

TRAPS			18		trap pending flags

WAIT			0		wait state flag

STOP_TRAPS		18		stop on trap flags

STOP_VECA		1		stop on read abort in trap or interrupt

STOP_SPA		1		stop on stack abort in trap or interrupt

PCQ[0:63]		16		PC prior to last jump, branch, or interrupt;

Most recent PC change first

WRU			8		interrupt character

The CPU attempts to detect when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE			enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  The CPU is considered idle if a WAIT instruction is executed.  This will work for RSTS/E and RSX-11M+, but not for RT-11 or UNIX.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 262144 entries.

### System Registers (SYSTEM)

The SYSTEM device implements registers that vary among CPU types:

name		models			size	comments

SR		11/04, 11/05, 11/20,	16	switch register or

11/23+, 11/34, 11/40,		configuration register

11/44, 11/45, 11/60,

11/70, 11/73B, 11/83,

11/84, 11/93, 11/94

DR		11/04, 11/05, 11/20,	16	display register or

1123+, 11/24, 11/34,		board LEDs

11/70, 11/73B, 11/83,

11/84, 11/93, 11/94

MEMERR	11/44, 11/60, 11/70,	16	memory error register

11/53, 11/73, 11/73B,

11/83, 11/84, 11/93,

11/94

CCR		11/44, 11/60, 11/70,	16	cache control register

11/53, 11/73, 11/73B,

11/83, 11/84, 11/93,

11/94

MAINT		11/23+, 11/44, 11/70,	16	maintenance register

11/53, 11/73, 11/73B,

11/83, 11/84, 11/93,

11/94

HITMISS	11/44, 11/60, 11/70,	16	hit/miss register

11/53, 11/73, 11/73B,

11/83, 11/84, 11/93,

11/94

CPUERR	11/24, 11/44, 11/70,	16	CPU error register

11/53, 11/73, 11/73B,

11/83, 11/84, 11/93,

11/94

MBRK		11/45, 11/70		16	microbreak register

SYSID		11/70				16	system ID (default = 1234 hex)

JCSR		11/53, 11/73B, 11/83,	16	board control/status

11/84, 11/93, 11/94

JPCR		11/23+, 11/53, 11/73B,	16	page control register

11/83, 11/84, 11/93,

11/94

JASR		11/93, 11/94		16	additional status

UDCR		11/84, 11/94		16	Unibus map diag control

UDDR		11/84, 11/94		16	Unibus map diag data

UCSR		11/84, 11/94		16	Unibus map control/status

ULAST		11/24				23	last Unibus map result

For the 11/83, 11/84, 11/93, and 11/94, the user can set the default value of the clock frequency:

SET SYSTEM JCLK_DEFAULT={LINE|50Hz|60HZ|800HZ}

The user can check the default value with the SHOW SYSTEM JCLK_DEFAULT command.

## I/O Devices

### Unibus and Qbus DMA Devices

DMA peripherals function differently, depending on whether the CPU type supports the Unibus or the Qbus, and whether the Unibus supports 22b direct memory access (11/70 with RH70 controllers):

peripheral	11/70		all		Qbus

+RH70		other

Unibus

CD		18b		18b		disabled

RC		18b		18b		disabled

RF		18b		18b		disabled

RK		18b		18b		disabled if mem > 256K

HK		18b		18b		disabled if mem > 256K

RL		18b		18b		22b RLV12

RP		22b		18b		22b third party

RQ		18b		18b		22b RQDX3

RY		18b		18b		disabled if mem > 256K

TC		18b		18b		disabled

TM		18b		18b		disabled if mem > 256K

TS		18b		18b		22b TSV05

TQ		18b		18b		22b TQK50

TU		22b		18b		22b third party

VH		18b		18b		22b DHQ11

XQ		disabled	disabled	22b DELQA

XU		18b		18b		disabled

Non-DMA peripherals work the same in all configurations.  Unibus-only peripherals are disabled in a Qbus configuration, and Qbus-only peripherals are disabled in a Unibus configuration.  In addition, Qbus DMA peripherals with only 18b addressing capability are disabled in a Qbus configuration with more than 256KB memory.

### I/O Device Addressing

PDP-11 I/O space and vector space are not large enough to allow all possible devices to be configured simultaneously at fixed addresses.  Instead, many devices have floating addresses and vectors; that is, the assigned device address and vector depend on the presence of other devices in the configuration:

DZ11			all instances have floating addresses

DHU11/DHQ11		all instances have floating addresses

RL11			first instance has fixed address, rest floating

RX11/RX211		first instance has fixed address, rest floating

DEUNA/DELUA		first instance has fixed address, rest floating

MSCP disk		first instance has fixed address, rest floating

TMSCP tape		first instance has fixed address, rest floating

In addition, some devices with fixed I/O space addresses have floating vector addresses.  DCI/DCO and DLI/DLO have floating vector addresses.

To maintain addressing consistency as the configuration changes, the simulator implements DEC's standard I/O address and vector autoconfiguration.  This allows the user to enable or disable devices without needing to manage I/O addresses and vectors.  For example, if RY is enabled while RX is present, RY is assigned an I/O address in the floating I/O space range; but if RX is disabled and then RY is enabled, RY is assigned the fixed "first instance" I/O address for floppy disks.

Autoconfiguration cannot solve address conflicts between devices with overlapping fixed addresses.  For example, with default I/O page addressing, the PDP-11 can support either a TM11 or a TS11, but not both, since they use the same I/O addresses.

In addition to autoconfiguration, most devices support the SET <device> ADDRESS command, which allows the I/O page address of the device to be changed, and the SET <device> VECTOR command, which allows the vector of the device to be changed.  Explicitly setting the I/O address of a device that normally uses autoconfiguration DISABLES autoconfiguration for that device and for the entire system.  As a consequence, the user may have to manually configure all other autoconfigured devices, because the autoconfiguration algorithm no longer recognizes the explicitly configured device.  A device can be reset to autoconfigure with the SET <device> AUTOCONFIGURE command.  Autoconfiguration can be restored for the entire system with the SET CPU AUTOCONFIGURE command.

The current I/O map can be displayed with the SHOW CPU IOSPACE command. Addresses that have set by autoconfiguration are marked with an asterisk (*).

All devices support the SHOW <device> ADDRESS and SHOW <device> VECTOR commands, which display the device address and vector, respectively.

## Programmed I/O Devices

### PC11 Paper Tape Reader (PTR)

The paper tape reader (PTR) reads data from a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper tape reader implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

BUSY			1		busy flag (CSR<11>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	     	  STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### PC11 Paper Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### DL11 Terminal Input (TTI)

The terminal interfaces (TTI, TTO) can be set to one of three modes, 7P, 7B or 8B:

mode	input characters		output characters

UC	high-order bit cleare,	high order-bit cleared,

lower case converted	lower case converted

to upper case		to upper case

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is 8B.

### DL11 Terminal Output (TTO)

The terminal input (TTI) polls the console keyboard for input.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters output

TIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the line clock)

The terminal output (TTO) writes to the simulator console window.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		time from I/O initiation to interrupt

### LP11 Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

### KW11-L Line-Time Clock (CLK)

The line-time clock (CLK) frequency can be adjusted as follows:

SET CLK 60HZ			set frequency to 60Hz

SET CLK 50HZ			set frequency to 50Hz

The default is 60Hz.

The line-time clock implements these registers:

name			size		comments

CSR			16		control/status register

INT			1		interrupt pending flag

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

TIME			24		clock interval

The line-time clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

### KW11-P Programmable Clock (PCLK)

The programmable clock (PCLK) line frequency can be adjusted as follows:

SET PCLK 60HZ			set frequency to 60Hz

SET PCLK 50HZ			set frequency to 50Hz

The default is 60Hz.

The programmable clock implements these registers:

name			size		comments

CSR			16		control/status register

CSB			16		count set buffer

CNT			16		current count

INT			1		interrupt pending flag

OVFL			1		overflow (error) flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

UPDN			1		up/down count mode (CSR<4>)

MODE			1		single/repeat mode (CSR<3>)

RUN			1		clock run (CSR<0>)

TIME[0 to 3]		32		clock interval, rates 0 to 3

TPS[0 to 3]		32		ticks per second, rates 0 to 3

The programmable clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time. Operation at the highest clock rate (100Khz) is not recommended.  The programmable

clock is disabled by default.

### TA11/TA60 Cassette Tape (CT)

The TA11 is a programmed I/O controller supporting two cassette drives (0 and 1).  The TA11 can be used like a small magtape under RT11 and RSX-11M, and with the CAPS-11 operating system.  Cassettes are simulated as magnetic tapes with a fixed capacity (93,000 characters).  The tape format is always SimH standard.  The TA11 is disabled by default.

TA11 options include the ability to make units write enabled or write locked.

SET CTn LOCKED			set unit n write locked

SET CTn WRITEENABLED		set unit n write enabled

Units can not be set ENABLED or DISABLED.  The TA11 does not support the BOOT command.

The TA11 controller implements these registers:

name			size		comments

TACS			16		control/status register

TAIDB			8		input data buffer

TAODB			8		output data buffer

INT			1		interrupt request

ERR			1		error flag

TR			1		transfer request flag

IE			1		interrupt enable flag

WRITE			1		TA60 write operation flag

BPTR			17		buffer pointer

BLNT			17		buffer length

STIME			24		operation start time

CTIME			24		character latency

STOP_IOE		1		stop on I/O errors flag

POS[0:1]		32		position, units 0-1

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	CRC error; if STOP_IOE, stop

## Floppy Disk Drives

### RX11/RX01 Floppy Disk (RX)

RX11 options include the ability to set units write enabled or write locked:

SET RXn LOCKED			set unit n write locked

SET RXn WRITEENABLED		set unit n write enabled

The RX11 supports the BOOT command.

The RX11 implements these registers:

name			size		comments

RXCS			12		status

RXDB			8		data buffer

RXES			8		error status

RXERR			8		error code

RXTA			8		current track

RXSA			8		current sector

STAPTR		3		controller state

BUFPTR		3		buffer pointer

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

TR			1		transfer ready flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

DONE			1		device done flag (CSR<5>)

CTIME			24		command completion time

STIME			24		seek time, per track

XTIME			24		transfer ready delay

STOP_IOE		1		stop on I/O error

SBUF[0:127]		8		sector buffer array

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### RX211/RX02 Floppy Disk (RY)

RX211 options include the ability to set units write enabled or write locked, single or double density, or autosized:

SET RYn LOCKED			set unit n write locked

SET RYn WRITEENABLED		set unit n write enabled

SET RYn SINGLE			set unit n single density

SET RYn DOUBLE			set unit n double density (default)

SET RYn AUTOSIZE			set unit n to autosize at ATTACH

The RX211 supports the BOOT command.  The RX211 is disabled in a Qbus system with more than 256KB of memory.

The RX211 implements these registers:

name			size		comments

RYCS			16		status

RYBA			16		buffer address

RYWC			8		word count

RYDB			16		data buffer

RYES			12		error status

RYERR			8		error code

RYTA			8		current track

RYSA			8		current sector

STAPTR		4		controller state

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

TR			1		transfer ready flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

DONE			1		device done flag (CSR<5>)

CTIME			24		command completion time

STIME			24		seek time, per track

XTIME			24		transfer ready delay

STOP_IOE		1		stop on I/O error

SBUF[0:255]		8		sector buffer array

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RX02 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## Cartridge Disk Drives

### RK11/RK05 Cartridge Disk (RK)

RK11 options include the ability to make units write enabled or write locked:

SET RKn LOCKED			set unit n write locked

SET RKn WRITEENABLED		set unit n write enabled

Units can also be set ENABLED or DISABLED.  The RK11 supports the BOOT command.  The RK11 is disabled in a Qbus system with more than 256KB of memory.

The RK11 implements these registers:

name			size		comments

RKCS			16		control/status

RKDA			16		disk address

RKBA			16		memory address

RKWC			16		word count

RKDS			16		drive status

RKER			16		error status

INTQ			9		interrupt queue

DRVN			3		number of last selected drive

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

INT			1		interrupt pending flag

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RK611/RK06,RK07 Cartridge Disk (HK)

RK611 options include the ability to set units write enabled or write locked, to set the drive type to RK06, RK07, or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET HKn LOCKED			set unit n write locked

SET HKn WRITEENABLED		set unit n write enabled

SET HKn RK06			set type to RK06

SET HKn RK07			set type to RK07

SET HKn AUTOSIZE			set type based on file size at ATTACH

SET HKn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RK611 supports the BOOT command.  The RK611 is disabled in a Qbus system with more than 256KB of memory.

The RK611 implements these registers:

name			size		comments

HKCS1			16		control/status 1

HKWC			16		word count

HKBA			16		bus address

HKDA			16		desired surface, sector

HKCS2			16		control/status 2

HKDS[0:7]		16		drive status, drives 0 to 7

HKER[0:7]		16		drive errors, drives 0 to 7

HKDB[0:2]		16		data buffer silo

HKDC			16		desired cylinder

HKOF			8		offset

HKMR			16		maintenance register

HKSPR			16		spare register

HKCI			1		controller interrupt flop

HKDI			1		drive interrupt flop

HKEI			1		error interrupt flop

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR1<7>)

IE			1		interrupt enable flag (CSR1<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

MIN2TIME		24		minimum time between DONE and ATA

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RL11(RLV12)/RL01,RL02 Cartridge Disk (RL)

RL11 options include the ability to set units write enabled or write locked, to set the drive type to RL01, RL02, or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET RLn LOCKED			set unit n write locked

SET RLn WRITEENABLED		set unit n write enabled

SET RLn RL01			set type to RL01

SET RLn RL02			set type to RL02

SET RLn AUTOSIZE			set type based on file size at ATTACH

SET RLn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RL11 supports the BOOT command.  In a Unibus system, the RL behaves like an RL11 with 18b addressing; in a Qbus (Q22) system, the RL behaves like the RLV12 with 22b addressing.

The RL11 implements these registers:

name			size		comments

RLCS			16		control/status

RLDA			16		disk address

RLBA			16		memory address

RLBAE			6		memory address extension (RLV12)

RLMP,RLMP1,RLMP2	16		multipurpose register queue

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

## Massbus Subsystems

### RH70/RH11 Massbus Adapters (RHA, RHB, RHC)

The RH70/RH11 Massbus adapters interface Massbus peripherals to the memory bus or Unibus of the CPU.  The simulator provides three Massbus adapters.  The first, RHA, is configured for the RP family of disk drives.  The second, RHB, is configured for the TU family of tape controllers.  The third, RHC, is configured for the RS family of fixed head disks.  By default, RHA is enabled, and RHB and RHC are disabled.  In a Unibus system, the RH adapters implement 22b addressing for the 11/70 and 18b addressing for all other models.  In a Qbus system, the RH adapters always implement 22b addressing.

Each RH adapter implements these registers:

name			size		comments

CS1			16		control/status register 1

WC			16		word count

BA			16		bus address

CS2			16		control/status register 2

DB			16		data buffer

BAE			6		bus address extension

CS3			16		control/status register 3

IFF			1		transfer complete interrupt request flop

INT			1		interrupt pending flag

SC			1		special condition (CSR1<15>)

DONE			1		device done flag (CSR1<7>)

IE			1		interrupt enable flag (CSR1<6>)

### RP04/05/06/07, RM02/03/05/80 Disk Pack Drives (RP)

The RP controller implements the Massbus family of large disk drives.  RP options include the ability to set units write enabled or write locked, to set the drive type to one of six disk types or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET RPn LOCKED			set unit n write locked

SET RPn WRITEENABLED		set unit n write enabled

SET RPn RM03			set type to RM03

SET RPn RM05			set type to RM05

SET RPn RM80			set type to RM80

SET RPn RP04			set type to RP04

SET RPn RP06			set type to RP06

SET RPn RP07			set type to RP07

SET RPn AUTOSIZE			set type based on file size at ATTACH

SET RPn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RP controller supports the BOOT command.

The RP controller implements the registers listed below.  Registers suffixed with [0:7] are replicated per drive.

name			size		comments

CS1[0:7]		16		current operation

DA[0:7]		16		desired surface, sector

DS[0:7]		16		drive status

ER1[0:7]		16		drive errors

OF[0:7]		16		offset

DC[0:7]		16		desired cylinder

ER2[0:7]		16		error status 2

ER3[0:7]		16		error status 3

EC1[0:7]		16		ECC syndrome 1

EC2[0:7]		16		ECC syndrome 2

MR[0:7]		16		maintenance register

MR2[0:7]		16		maintenance register 2 (RM only)

HR[0:7]		16		holding register (RM only)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### TM02/TM03/TE16/TU45/TU77 Magnetic Tapes (TU)

The TU controller implements the Massbus family of 800/1600bpi magnetic tape drives.  TU options include the ability to select the formatter type (TM02 or TM03), to set the drive type to one of three drives (TE16, TU45, or TU77), and to set the drives write enabled or write locked.

SET TU TM02				set controller type to TM02

SET TU TM03				set controller type to TM03

SET TUn TE16			set drive type to TE16

SET TUn TU45			set drive type to TU45

SET TUn TU77			set drive type to TU77

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TUn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TUn CAPAC			show unit n capacity in MB

Units can be set ENABLED or DISABLED.  The TU controller supports the BOOT command.

The TU controller implements the following registers:

name			size		comments

CS1			6		current operation

FC			16		frame count

FS			16		formatter status

ER			16		formatter errors

CC			16		check character

MR			16		maintenance register

TC			16		tape control register

TIME			24		operation execution time

UST			17		unit status, drives 0 to 7

POS			32		position, drive 0 to 7

STOP_IOE		1		stop of I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	parity error; if STOP_IOE, stop

### RS03/RS04 Fixed Head Disks

The RS controller implements the Massbus family fixed head disks.  RS options include the ability to set units write enabled or write locked and to set the drive type to RS03, RS04, or autosize.

SET RSn LOCKED			set unit n write lock enabled

SET RSn WRITEENABLED		set unit n write enabled

SET RSn RS03			set type to RS03

SET RSn RS04			set type to RS04

SET RSn AUTOSIZE			set type based on file size at ATTACH

The drive type options can be used only when a unit is not attached to a file.  Units can be set ENABLED or DISABLED.  The RS controller supports the BOOT command.

The RS controller implements the registers listed below.  Registers suffixed with [0:7] are replicated per drive.

name			size		comments

CS1[0:7]		16		current operation

DA[0:7]		16		desired track, sector

DS[0:7]		16		drive status

ER[0:7]		16		drive errors

MR[0:7]		16		maintenance register

WLK[0:7]		6		max write locked track, if enabled

TIME			24		rotational delay, per word

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RS data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## RQDX3/UDA50 MSCP Disk Controllers (RQ, RQB, RQC, RQD)

The simulator implements four MSCP disk controllers, RQ, RQB, RQC, RQD.  Initially, RQB, RQC, and RQD are disabled.  Each RQ controller simulates an RQDX3 MSCP disk controller with four disk drives.  RQ options include the ability to set units write enabled or write locked, and to set the drive type to one of many disk types:

SET RQn LOCKED			set unit n write locked

SET RQn WRITEENABLED		set unit n write enabled

SET RQn RX50			set type to RX50

SET RQn RX33			set type to RX33

SET RQn RD32			set type to RD32

SET RQn RD51			set type to RD51

SET RQn RD52			set type to RD52

SET RQn RD53			set type to RD53

SET RQn RD54			set type to RD54

SET RQn RD31			set type to RD31

SET RQn RA81			set type to RA81

SET RQn RA82			set type to RA82

set RQn RA71			set type to RA71

SET RQn RA72			set type to RA72

SET RQn RA90			set type to RA90

SET RQn RA92			set type to RA92

SET RQn RRD40			set type to RRD40 (CD ROM)

SET RQn RAUSER{=n}		set type to RA82 with n MB's

SET -L RQn RAUSER{=n}		set type to RA82 with n LBN's

The type options can be used only when a unit is not attached to a file.  RAUSER is a "user specified" disk; the user can specify the size of the disk in either MB (1000000 bytes) or logical block numbers (LBN's, 512 bytes each).  The minimum size is 5MB; the maximum size is 2GB without extended file support, 1TB with extended file support.

Units can be set ENABLED or DISABLED.  Each RQ controller supports the BOOT command.  In a Unibus system, an RQ supports 18b addressing and identifies itself as a UDA50.  In a Qbus system, an RQ supports 22b addressing and identifies itself as an RQDX3.

Each RQ controller implements the following special SHOW commands:

SHOW RQn TYPE			show drive type

SHOW RQ RINGS			show command and response rings

SHOW RQ FREEQ			show packet free queue

SHOW RQ RESPQ			show packet response queue

SHOW RQ UNITQ			show unit queues

SHOW RQ ALL				show all ring and queue state

SHOW RQn UNITQ			show unit queues for unit n

Each RQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

Some DEC operating systems, notably RSX11M/M+, are very sensitive to the timing parameters.  Changing the default values may cause M/M+ to crash on boot or to hang during operation.

Error handling is as follows:

error	     		processed as

not attached	disk not ready

end of file		assume rest of disk is zero

OS I/O error	report error and stop

## Fixed Head Disks

### RC11 Fixed Head Disk (RC)

RC11 options include the ability to set the number of platters to a fixed value between 1 and 4, or to autosize the number of platters:

SET RC 1P				one platter (256K)

SET RC 2P				two platters (512K)

SET RC 3P				three platters (768K)

SET RC 4P				four platters (1024K)

SET RC AUTOSIZE			autosized on ATTACH

The default is one platter.  The RC11 does not support the BOOT command.  The RC11 is disabled at startup and is automatically disabled in a Qbus system.

The RC11 is a DMA device.  The entire transfer occurs in a single DMA transfer.

The RC11 implements these registers:

name			size		comments

RCLA			16		look ahead register

RCDA			16		current disk address

RCER			16		error register

RCCS			16		control/status

RCWC			16		word count

RCCA			16		current memory address

RCMN			16		maintenance register

RCDB			16		data buffer

RCWLK			32		write lock switches

INT			1		interrupt pending flag

ERR			1		device error flag

DONE			1		device done flag

IE			1		interrupt enable flag

TIME			24		rotational delay, per word

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		non-existent disk

RC11 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### RF11/RS11 Fixed Head Disk (RF)

RF11 options include the ability to set the number of platters to a fixed value between 1 and 8, or to autosize the number of platters:

SET RF 1P				one platter (256K)

SET RF 2P				two platters (512K)

SET RF 3P				three platters (768K)

SET RF 4P				four platters (1024K)

SET RF 5P				five platters (1280K)

SET RF 6P				six platters (1536K)

SET RF 7P				seven platters (1792K)

SET RF 8P				eight platters (2048K)

SET RF AUTOSIZE			autosized on ATTACH

The default is one platter.  The RF11 supports the BOOT command.  The RF11 is disabled at startup and is automatically disabled in a Qbus system.

The RF11 implements these registers:

name			size		comments

RFCS			16		control/status

RFWC			16		word count

RFCMA			16		current memory address

RFDA			16		current disk address

RFDAE			16		disk address extension

RFDBR			16		data buffer

RFMR			16		maintenance register

RFWLK			32		write lock switches

INT			1		interrupt pending flag

ERR			1		device error flag

DONE			1		device done flag

IE			1		interrupt enable flag

TIME			24		rotational delay, per word

BURST			1		burst flag

STOP_IOE		1		stop on I/O error

The RF11 is a DMA device.  If BURST = 0, word transfers are scheduled individually; if BURST = 1, the entire transfer occurs in a single DMA transfer.

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		non-existent disk

RF11 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## TC11/TU56 DECtape (DT)

The DT controller implements the TC11 DECtape controller and TU56 drives.  DECtape options include the ability to make units write enabled or write locked.

SET DTn LOCKED			set unit n write locked

SET DTn WRITEENABLED		set unit n write enabled

Units can be set ENABLED or DISABLED.  The TC11 supports the BOOT command.  The TC11 is automatically disabled in a Qbus system.

The TC11 supports supports PDP-8 format, PDP-11 format, and 18b format DECtape images.  ATTACH assumes the image is in PDP-11 format; the user can force other choices with switches:

-t					PDP-8 format

-f					18b format

-a					autoselect based on file size

The DECtape controller is a data-only simulator; the timing and mark track, and block header and trailer, are not stored.  Thus, the WRITE TIMING AND MARK TRACK function is not supported; the READ ALL function

always returns the hardware standard block header and trailer; and the WRITE ALL function dumps non-data words into the bit bucket.

The TC controller implements these registers:

name			size		comments

TCST			16		status register

TCCM			16		command register

TCWC			16		word count register

TCBA			16		bus address register

TCDT			16		data register

INT			1		interrupt pending flag

ERR			1		error flag

DONE			1		done flag

IE			1		interrupt enable flag

CTIME			31		time to complete transport stop

LTIME			31		time between lines

DCTIME		31		time to decelerate to a full stop

SUBSTATE		2		read/write command substate

POS[0:7]		32		position, in lines, units 0 to 7

STATT[0:7]		31		unit state, units 0 to 7

STOP_OFFR		1		stop on off-reel error

It is critically important to maintain certain timing relationships among the DECtape parameters, or the DECtape simulator will fail to operate correctly.

- LTIME must be at least 6

- DCTIME needs to be at least 100 times LTIME

Acceleration time is set to 75% of deceleration time.

## Magnetic Tape Controllers

### TM11 Magnetic Tape (TM)

TM options include the ability to make units write enabled or write locked.

SET TMn LOCKED			set unit n write locked

SET TMn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TMn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TMn CAPAC			show unit n capacity in MB

Units can be set ENABLED or DISABLED.

The TM11 supports the BOOT command.  The bootstrap supports both original and DEC standard boot formats.  Originally, a tape bootstrap read and executed the first record on tape.  To allow for ANSI labels, the DEC standard bootstrap skipped the first record and read and executed the second.  The DEC standard is the default; to bootstrap an original format tape, use the command BOOT –O MTn.  The TM11 is automatically disabled in a Qbus system with more than 256KB of memory.

The TM controller implements these registers:

name			size		comments

MTS			16		status

MTC			16		command

MTCMA			16		memory address

MTBRC			16		byte/record count

INT			1		interrupt pending flag

ERR			1		error flag

DONE			1		device done flag

IE			1		interrupt enable flag

STOP_IOE		1		stop on I/O error

TIME			24		delay

UST[0:7]		16		unit status, units 0 to 7

POS[0:7]		32		position, units 0 to 7

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	parity error; if STOP_IOE, stop

### TS11/TSV05 Magnetic Tape (TS)

TS options include the ability to make the unit write enabled or write locked.

SET TS LOCKED			set unit write locked

SET TS WRITEENABLED		set unit write enabled

The TS drive can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TS0 CAPAC=m			set capacity to m MB (0 = unlimited)

SHOW TS0 CAPAC			show capacity in MB

The TS11 supports the BOOT command.  The bootstrap supports only DEC standard boot formats.  To allow for ANSI labels, the DEC standard bootstrap skipped the first record and read and executed the second.  In a Unibus system, the TS behaves like the TS11 and implements 18b addresses.  In a Qbus system, the TS behaves like the TSV05 and implements 22b addresses.

The TS controller implements these registers:

name			size		comments

TSSR			16		status register

TSBA			16		bus address register

TSDBX			16		data buffer extension register

CHDR			16		command packet header

CADL			16		command packet low address or count

CADH			16		command packet high address

CLNT			16		command packet length

MHDR			16		message packet header

MRFC			16		message packet residual frame count

MXS0			16		message packet extended status 0

MXS1			16		message packet extended status 1

MXS2			16		message packet extended status 2

MXS3			16		message packet extended status 3

MXS4			16		message packet extended status 4

WADL			16		write char packet low address

WADH			16		write char packet high address

WLNT			16		write char packet length

WOPT			16		write char packet options

WXOPT			16		write char packet extended options

ATTN			1		attention message pending

BOOT			1		boot request pending

OWNC			1		if set, tape owns command buffer

OWNM			1		if set, tape owns message buffer

TIME			24		delay

POS			32		position

Error handling is as follows:

error			processed as

not attached	tape not ready

end of file		bad tape

OS I/O error	fatal tape error

### TQK50 TMSCP Disk Controller (TQ)

The TQ controller simulates the TQK50 TMSCP disk controller.  TQ options include the ability to set units write enabled or write locked, and to specify the controller type and tape length:

SET TQn LOCKED			set unit n write locked

SET TQn WRITEENABLED		set unit n write enabled

SET TQ TK50				set controller type to TK50

SET TQ TK70				set controller type to TK70

SET TQ TU81				set controller type to TU81

SET TQ TKUSER{=n}			set controller type to TK50 with

tape capacity of n MB

User-specified capacity must be between 50 and 2000 MB.

Regardless of the controller type, individual units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TQn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TQn CAPAC			show unit n capacity in MB

The TQ controller supports the BOOT command.   In a Unibus system, the TQ supports 18b addressing.  In a Qbus system, the TQ supports 22b addressing.

The TQ controller implements the following special SHOW commands:

SHOW TQ TYPE			show controller type

SHOW TQ RINGS			show command and response rings

SHOW TQ FREEQ			show packet free queue

SHOW TQ RESPQ			show packet response queue

SHOW TQ UNITQ			show unit queues

SHOW TQ ALL				show all ring and queue state

SHOW TQn UNITQ			show unit queues for unit n

The TQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

POS[0:3]		32		tape position, units 0 to 3

OBJP[0:3]		32		object position, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

Some DEC operating systems, notably RSX11M/M+, are very sensitive to the timing parameters.  Changing the default values may cause M/M+ to crash on boot or to hang during operation.

Error handling is as follows:

error	     		processed as

not attached	tape not ready

end of file		end of medium

OS I/O error	fatal tape error

## Communications Devices

### DC11 Additional Terminal Interfaces (DCI/DCO)

For very early system programs, the PDP-11 simulator supports up to sixteen additional DC11 terminal interfaces.  The additional terminals consist of two independent devices, DCI and DCO.  The entire set is modeled as a terminal multiplexer, with DCI as the master controller.  The additional terminals perform input and output through Telnet sessions connected to a user-specified port.  The number of lines is specified with a SET command:

SET DCIX LINES=n			set number of additional lines to n [1-16]

The ATTACH command specifies the port to be used:

ATTACH DCIX <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The additional terminals are disabled by default.

The additional terminals can be set to one of four modes: UC, 7P, 7B, or 8B.

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is 7P.  In addition, each line can be configured to behave as though it was attached to a dataset, or hardwired to a terminal:

SET DCOn DATASET			simulate attachment to a dataset (modem)

SET DCOn NODATASET		simulate direct attachment to a terminal

Finally, each line supports output logging.  The SET DCOn LOG command enables logging on a line:

SET DCOn LOG=filename		log output of line n to filename

The SET DCOn NOLOG command disables logging and closes the open log file, if any.

Once DCI is attached and the simulator is running, the terminals listen for connections on the specified port.  They assume that the incoming connections are Telnet connections.  The connections remain open until disconnected either by the Telnet client, a SET DCI DISCONNECT command, or a DETACH DCI command.

Other special commands:

SHOW DCI CONNECTIONS 		show current connections

SHOW DCI STATISTICS 		show statistics for active connections

SET DCOn DISCONNECT		disconnects the specified line.

The input device  (DCI) implements these registers:

name			size		comments

CSR[0:15]		16		input control/stats register, lines 0 to 15

BUF[0:15]		16		input buffer, lines 0 to 15

IREQ			16		interrupt requests, lines 0 to 15

The output device (DCO) implements these registers:

name			size		comments

CSR[0:15]		16		input control/stats register, lines 0 to 15

BUF[0:15]		8		input buffer, lines 0 to 15

IREQ			16		interrupt requests, lines 0 to 15

TIME[0:15]		31		time from I/O initiation to interrupt,

lines 0 to 15

The additional terminals do not support save and restore.  All open connections are lost when the simulator shuts down or DCI is detached.

### KL11/DL11 Additional Terminal Interfaces (DLI/DLO)

The PDP-11 simulator supports up to sixteen additional KL11/DL11 terminal interfaces.  The additional terminals consist of two independent devices, DLI and DLO.  The entire set is modeled as a terminal multiplexer, with DLI as the master controller.  The additional terminals perform input and output through Telnet sessions connected to a user-specified port.  The number of lines is specified with a SET command:

SET DLI LINES=n			set number of additional lines to n [1-16]

The ATTACH command specifies the port to be used:

ATTACH DLI <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The additional terminals are disabled by default.

The additional terminals can be set to one of four modes: UC, 7P, 7B, or 8B.

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is 7P.  .  In addition, each line can be configured to behave as though it was attached to a dataset, or hardwired to a terminal:

SET DLOn DATASET			simulate attachment to a dataset (modem)

SET DLOn NODATASET		simulate direct attachment to a terminal

Finally, each line supports output logging.  The SET DLOn LOG command enables logging on a line:

SET DLOn LOG=filename		log output of line n to filename

The SET DLOn NOLOG command disables logging and closes the open log file, if any.

Once DLI is attached and the simulator is running, the terminals listen for connections on the specified port.  They assume that the incoming connections are Telnet connections.  The connections remain open until disconnected either by the Telnet client, a SET DLI DISCONNECT command, or a DETACH DLI command.

Other special commands:

SHOW DLI CONNECTIONS 		show current connections

SHOW DLI STATISTICS 		show statistics for active connections

SET DLOn DISCONNECT		disconnects the specified line.

The input device  (DLI) implements these registers:

name			size		comments

CSR[0:15]		16		input control/stats register, lines 0 to 15

BUF[0:15]		16		input buffer, lines 0 to 15

IREQ			16		receive interrupt requests, lines 0 to 15

DSI			16		dataset interrupt requests, lines 0 to 15

The output device (DLO) implements these registers:

name			size		comments

CSR[0:15]		16		input control/stats register, lines 0 to 15

BUF[0:15]		8		input buffer, lines 0 to 15

IREQ			16		interrupt requests, lines 0 to 15

TIME[0:15]		31		time from I/O initiation to interrupt,

lines 0 to 15

The additional terminals do not support save and restore.  All open connections are lost when the simulator shuts down or DLO is detached.

### DZ11 Terminal Multiplexer (DZ)

The DZ11 is an 8-line terminal multiplexer.  Up to 4 DZ11's (32 lines) are supported.  The number of lines can be changed with the command

SET DZ LINES=n			set line count to n

The line count must be a multiple of 8, with a maximum of 32.

The DZ11 supports three character processing modes, 7P, 7B, and 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default is 8B.

The DZ11 supports logging on a per-line basis.  The command

SET DZ LOG=line=filename

enables logging for the specified line to the indicated file.  The command

SET DZ NOLOG=line

disables logging for the specified line and closes any open log file.  Finally, the command

SHOW DZ LOG

displays logging information for all DZ lines.

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH {-am} DZ <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The optional switch -m turns on the DZ11's modem controls; the optional switch -a turns on active disconnects (disconnect session if computer clears Data Terminal Ready).  Without modem control, the DZ behaves as though terminals were directly connected; disconnecting the Telnet session does not cause any operating system-visible change in line status.

Once the DZ is attached and the simulator is running, the DZ will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET DZ DISCONNECT command, or a DETACH DZ command.

Other special DZ commands:

SHOW DZ CONNECTIONS 		show current connections

SHOW DZ STATISTICS 		show statistics for active connections

SET DZ DISCONNECT=linenumber	disconnects the specified line.

The DZ11 implements these registers:

name			size		comments

CSR[0:3]		16		control/status register, boards 0 to 3

RBUF[0:3]		16		receive buffer, boards 0 to 3

LPR[0:3]		16		line parameter register, boards 0 to 3

TCR[0:3]		16		transmission control register, boards 0 to 3

MSR[0:3]		16		modem status register, boards 0 to 3

TDR[0:3]		16		transmit data register, boards 0 to 3

SAENB[0:3]		1		silo alarm enabled, boards 0 to 3

RXINT			4		receive interrupts, boards 3..0

TXINT			4		transmit interrupts, boards 3..0

MDMTCL		1		modem control enabled

AUTODS		1		autodisconnect enabled

The DZ11 does not support save and restore.  All open connections are lost when the simulator shuts down or the DZ is detached.

### DHQ11 Terminal Multiplexer (VH)

The DHQ11 is an 8-line terminal multiplexer for Qbus systems.  Up to 4 DHQ11's are supported.

The DHQ11 is a programmable asynchronous terminal multiplexer.  It has two programming modes: DHV11 and DHU11.  The register sets are compatible with these devices.  For transmission, the DHQ11 can be used in either DMA or programmed I/O mode.  For reception, there is a 256-entry FIFO for received characters, dataset status changes, and diagnostic information, and a programmable input interrupt timer (in DHU mode).  The device supports 16-, 18-, and 22-bit addressing.  The DHQ11 can be programmed to filter and/or handle XON/XOFF characters independently of the processor.  The DHQ11 supports programmable bit width (between 5 and 8) for the input and output of characters.

The DHQ11 has a rocker switch for determining the programming mode.  By default, the DHV11 mode is selected, though DHU11 mode is recommended for applications that can support it.  The VH controller may be adjusted on a per controller basis as follows:

SET VHn DHU				use the DHU programming mode and registers

SET VHn DHV				use the DHV programming mode and registers

DMA output is supported.  In a real DHQ11, DMA is not initiated immediately upon receipt of TX.DMA.START but is dependent upon some internal processes.  The VH controller mimics this behavior by default.  It may be desirable to alter this and start immediately, though this may not be compatible with all operating systems and diagnostics.  You can change the behavior of the VH controller as follows:

SET VHn NORMAL			use normal DMA procedures

SET VHn FASTDMA			set DMA to initiate immediately

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH VH <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not  being used for other TCP/IP activities.  This port is the point of entry for all lines on all VH controllers.

The number of lines can be changed with the command

SET VH LINES=n			set line count to n

The line count must be a multiple of 8, with a maximum of 32.

Modem and auto-disconnect support may be set on an individual controller basis.  The SET MODEM command directs the controller to report modem status changes to the computer.  The SET HANGUP command turns on active disconnects (disconnect session if computer clears Data Terminal Ready).

SET VHn [NO]MODEM			disable/enable modem control

SET VHn [NO]HANGUP		disable/enable disconnect on DTR drop

Once the VH is attached and the simulator is running, the VH will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET VH DISCONNECT command, or a DETACH VH command.

Other special VH commands:

SHOW VH CONNECTIONS 		show current connections

SHOW VH STATISTICS 		show statistics for active connections

SET VH DISCONNECT=linenumber	disconnects the specified line.

The DHQ11 implements these registers, though not all can be examined from SCP:

name            size    	comments

CSR[0:3]        16      	control/status register, boards 0 to 3

RBUF[0:3]       16      	receive buffer, boards 0 to 3

LPR[0:3]        16      	line parameter register, boards 0 to 3

RXINT           4       	receive interrupts, boards 3..0

TXINT           4       	transmit interrupts, boards 3..0

[more to be described...]

The DHQ11 does not support save and restore.  All open connections are lost when the simulator shuts down or the VH is detached.

## Ethernet Controllers

### DELQA-T/DELQA/DEQNA Qbus Ethernet Controllers (XQ, XQB)

The simulator implements two DELQA-T/DELQA/DEQNA Qbus Ethernet controllers (XQ, XQB).  Initially, XQ is enabled, and XQB is disabled.  Options allow control of the MAC address, the controller mode, and the sanity timer.

SET  XQ MAC=<mac-address>	ex. 08-00-2B-AA-BB-CC

SHOW XQ MAC

These commands are used to change or display the MAC address.  <mac-address> is a valid Ethernet MAC, delimited by dashes or periods.  The controller defaults to 08-00-2B-AA-BB-CC, which should be sufficient if there is only one SIMH controller on your LAN.  Two cards with the same MAC address will see each other's packets, resulting in a serious mess.

SET  XQ TYPE={DEQNA|[DELQA]|DELQA-T}

SHOW XQ TYPE

These commands are used to change or display the controller mode. DELQA mode is better and faster but may not be usable by older or non-DEC OS's.  Also, be aware that DEQNA mode is not supported by many modern OS's. The DEQNA-LOCK mode of the DELQA card is emulated by setting the the controller to DEQNA -- there is no need for a separate mode.  DEQNA-LOCK mode behaves exactly like a DEQNA, except for the operation of the VAR and MOP processing.

SET  XQ SANITY={ON|[OFF]}

SHOW XQ SANITY

These commands change or display the INITIALIZATION sanity timer (DEQNA jumper W3/DELQA switch S4).  The INITIALIZATION sanity timer has a default timeout of 4 minutes, and cannot be turned off, just reset.  The normal sanity timer can be set by operating system software regardless of the state of this switch.  Note that only the DEQNA (or the DELQA in DEQNA-LOCK mode (=DEQNA)) supports the sanity timer -- it is ignored by a DELQA in Normal mode, which uses switch S4 for a different purpose.

SET  XQ POLL={DEFAULT|4..2500}

SHOW XQ POLL

These commands change or display the service polling timer.  The polling timer is calibrated to run the service thread 200 times per second.  This value can be changed to accommodate particular system requirements for more (or less) frequent polling.

SHOW XQ STATS

This command will display the accumulated statistics for the simulated Ethernet controller.

To access the network, the simulated Ethernet controller must be attached to a real Ethernet interface:

ATTACH XQ0 {ethX|<device_name>}		ex. eth0 or /dev/era0

SHOW XQ ETH

where X in 'ethX' is the number of the Ethernet controller to attach, or the real device name.  The X number is system-dependent.  If you only have one Ethernet controller, the number will probably be 0. To find out what your system thinks the Ethernet numbers are, use the SHOW XQ ETH command.  The device list can be quite cryptic, depending on the host system, but is probably better than guessing. If you do not attach the device, the controller will behave as though the Ethernet cable were unplugged.

XQ and XQB have the following registers:

name			size		comments

SA0			16		station address word 0

SA1			16		station address word 1

SA2			16		station address word 2

SA3			16		station address word 3

SA4			16		station address word 4

SA5			16		station address word 5

RBDL			32		receive buffer descriptor list

XBDL			32		trans(X)mit buffer descriptor list

CSR			16		control status register

VAR			16		vector address register

INT			1		interrupt request flag

One final note: because of its asynchronous nature, the XQ controller is not limited to the ~1.5Mbit/sec of the real DEQNA/DELQA controllers, nor the 10Mbit/sec of a standard Ethernet.  Attach it to a Fast Ethernet (100 Mbit/sec) card, and "Feel the Power!" :-)

### DELUA/DEUNA Unibus Ethernet Controllers (XU, XUB)

The simulator implements two DELUA/DEUNA Unibus Ethernet controllers (XU, XUB).  Its operation is analogous to the DELQA/DEQNA controller.

## CR11/CD11 Card Reader (CR)

The card reader (CR) implements a single controller (either the CR11 or the CD11) and card reader (e.g., Documation M200, GDI Model 100) by reading a file and presenting lines or cards to the simulator.  Card decks may be represented by plain text ASCII files, card image files, or column binary files.  The CR11 controller is also compatible with the CM11-F, CME11, and CMS11.

Card image files are a file format designed by Douglas W. Jones at the University of Iowa to support the interchange of card deck data.  These files have a much richer information carrying capacity than plain ASCII files.  Card Image files can contain such interchange information as card-stock color, corner cuts, special artwork, as well as the binary punch data representing all 12 columns.  Complete details on the format, as well as sample code, are available at Prof. Jones's site: http://www.cs.uiowa.edu/~jones/cards/.

The card reader can be configured to support either of the two controllers supported by DEC:

SET CR CR11		set controller type to CR11

SET CR CD11		set controller type to CD11

The controller type must be set before attaching a virtual card deck to the device.  You may NOT change controller type once a file is attached.

The primary differences are summarized in the table below.  By default, the CR11 simulation is selected.

CR11			CD11

BR			6			4

registers		4			3

data transfer	BR			DMA

card rate		200-600		1000-1200

hopper cap.		<= 1000		1000-2250

cards			Mark-sense &	punched only

punched

Examples of the CR11 include the M8290 and M8291 (CMS11).  All card readers use a common vector at 0230 and CSR at 177160.

The card reader supports ASCII, card image, and column binary format card “decks.”  When reading plain ASCII files, lines longer than 80 characters are silently truncated.  Card image support is included for 80 column Hollerith, 82 column Hollerith (silently ignoring columns 0 and 81), and 40 column Hollerith (mark-sense) cards.  Column binary supports 80 column card images only.  All files are attached read-only (as if the -R switch were given).

ATTACH –A CR <file>			file is ASCII text

ATTACH –B CR <file>			file is column binary

ATTACH –I CR <file>			file is card image format

If no flags are given, the file extension is evaluated.  If the filename ends in .TXT, the file is treated as ASCII text.  If the filename ends in .CBN, the file is treated as column binary.  Otherwise, the CR driver looks for a card image header.  If a correct header is found the file is treated as card image format, otherwise it is treated as ASCII text.

The correct character translation MUST be set if a plain text file is to be used for card deck input.  The correct translation SHOULD be set to allow correct ASCII debugging of a card image or column binary input deck.  Depending upon the operating system in use, how it was generated, and how the card data will be read and used, the translation must be set correctly so that the proper character set is used by the driver.  Use the following command to explicitly set the correct translation:

SET TRANSLATION={DEFAULT|026|026FTN|029|EBCDIC}

This command should be given after a deck is attached to the simulator.  The mappings above are completely described at http://www.cs.uiowa.edu/~jones/cards/codes.html.  Note that DEC typically used 029 or 026FTN mappings.

DEC operating systems used a variety of methods to determine the end of a deck, recognizing that 'hopper empty' does not necessarily mean the end of a deck.  Below is a summary of the various operating system conventions for signaling end of deck:

RT-11:	12-11-0-1-6-7-8-9 punch in column 1

RSTS/E:	12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

RSX:		12-11-0-1-6-7-8-9 punch

VMS:		12-11-0-1-6-7-8-9 punch in first 8 columns

TOPS:		12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

Using the AUTOEOF setting, the card reader can be set to automatically generate an EOF card consisting of the 12-11-0-1-6-7-8-9 punch in columns 1-8.  When set to CD11 mode, this switch also enables automatic setting of the EOF bit in the controller after the EOF card has been processed.  [The CR11 does not have a similar capability.]  By default AUTOEOF is enabled.

SET CR AUTOEOF

SET CR NOAUTOEOF

The default card reader rate for the CR11 is 285 cpm, while the default rate for the CD11 is 1000 cpm.  The reader rate can be set to its default value or to anywhere in the range 200..1200 cpm.  This rate may be changed while the unit is attached.

SET CR RATE={DEFAULT|200..1200}

It is standard operating procedure for operators to load a card deck and press the momentary action RESET button to clear any error conditions and alert the processor that a deck is available to read.  Use the following command to simulate pressing the card reader RESET button,

SET CR RESET

Another common control of physical card readers is the STOP button.  An operator could use this button to finish the read operation for the current card and terminate reading a deck early.  Use the following command to simulate pressing the card reader STOP button.

SET CR STOP

The simulator does not support the BOOT command.  The simulator does not stop on file I/O errors.  Instead the controller signals a reader check to the CPU.

The CR controller implements these registers:

name			size		comments

BUF			8		ASCII value of last column processed

CRS			16		CR11 status register

CRB1			16		CR11 12-bit Hollerith character

CRB2			16		CR11 8-bit compressed character

CRM			16		CR11 maintenance register

CDST			16		CD11 control/status register

CDCC			16		CD11 column count

CDBA			16		CD11 current bus address

CDDB			16		CD11 data buffer, 2nd status

BLOWER		2		blower state value

INTCR			1		interrupt pending flag (CR11)

INTCD			1		interrupt pending flag (CD11)

ERR			1		error flag (CRS<15>)

IE			1		interrupt enable flag (CRS<6>)

POS			32		file position - do not alter

TIME			24		delay time between columns

The CD11 simulation includes the Rev. J modification to make the CDDB act as a second status register during non-data transfer periods.

## Arithmetic Options

### KE11A Extended Arithmetic Option (KE)

The KE11A extended arithmetic option (KE) provides multiply, divide, normalization, and multi-bit shift capability on Unibus PDP-11’s that lack the EIS instruction set.  In practice, it was only sold with the PDP-11/20.  The KE is disabled by default.

The KE implements these registers:

name			size		comments

AC			16		accumulator

MQ			16		multiplier-quotient

SC			6		shift count

SR			8		status register

### KG11A Communications Arithmetic Option (KG)

The KG11-A is a programmed I/O, non-interrupting, dedicated arithmetic processor for the Unibus.  The device is used to compute the block check character (BCC) over a block of data, typically in data communication applications.  The KG11 can compute three different Cyclic Redundancy Check (CRC) polynomials (CRC-16, CRC-12, CRC-CCITT) and two Longitudinal Redundancy Checks (LRC, Exclusive-OR; LRC-8, LRC-16).  Up to eight units may be contiguously present in a single machine and are all located at fixed addresses.  This simulation implements all functionality of the device including the ability to single step computation of the BCC.   The KG is disabled by default.

The KG11 supports the following options: 

        SET KG UNITS=n           	set the number of units [0-8] 
        SET KG DEBUG={opt,opt…}   	set the debugging options 
                                    REG - any time a register is touched 
                                    POLY - any time the polynomial is changed 
                                    CYCLE - each cycle computing the polynomial 

The KG11 implements the following registers, replicated for each unit: 

        name            size        comments 

        SR[0:7]         16          control and status register; R/W 
        BCC[0:7]        16          result block check character; R/O 
        DR[0:7]         16          input data register; W/O 
        PULSCNT[0:7]    16          polynomial cycle stage

# Symbolic Display and Input

The PDP-11 simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as ASCII character

-c			display as two packed ASCII characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		two packed ASCII characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses standard PDP-11 assembler syntax.  There are sixteen instruction classes:

class			operands			examples			comments

no operands		none				HALT, RESET

3b literal		literal [0 to 7]		SPL

6b literal		literal [0-077]		MARK

8b literal		literal [0-0377]		EMT, TRAP

register		register			RTS

sop			specifier			SWAB, CLR, ASL

reg-sop		register, specifier	JSR, XOR, MUL

fop			flt specifier		ABSf, NEGf

ac-fop		flt reg, flt specifier	LDf, MULf

ac-sop		flt reg, specifier	LDEXP, STEXP

ac-moded sop	flt reg, specifier	LDCif, STCfi

dop			specifier, specifier	MOV, ADD, BIC

cond branch		address			BR, BCC, BNE

sob			register, address		SOB

cc clear		cc clear instructions	CLC, CLV, CLZ, CLN	combinable

cc set		cc set instructions	SEC, SEV, SEZ, SEN	combinable

For floating point opcodes, F and D variants, and I and L variants, may be specified regardless of the state of FPS.

The syntax for specifiers is as follows:

syntax		specifier	displacement	comments

Rn			0n		-

Fn			0n		-			only in flt reg classes

(Rn)			1n		-

@(Rn)			7n		0			equivalent to @0(Rn)

(Rn)+			2n		-

@(Rn)+		3n          -

-(Rn)			4n          -

@-(Rn)		5n          -

{+/-}d(Rn)		6n		{+/-}d

@{+/-}d(Rn)		7n		{+/-}d

#n			27		n

@#n			37		n

.+/-n			67		+/-n - 4

@.+/-n		77		+/-n - 4

{+/-}n		67		{+/-}n - PC - 4	if on disk, 37 and n

@{+/-}n		77		{+/-}n - PC - 4	if on disk, invalid

# The UC15

The UC15 is a special, limited configuration of the PDP-11 simulator for use as the I/O processor in a PDP-15/76 system. It is configured as follows:

device name(s)	simulates

CPU			PDP-11/05 CPU with 8KB-24KB of memory

PTR,PTP		PC11 paper tape reader/punch

TTI,TTO		DL11 console terminal

CR			CR11 card reader

LPT			LP11 line printer

CLK			KW11-L line frequency clock

RK			RK11/RK05 cartridge disk controller with eight drives

UCA,UCB		DR11-C parallel interfaces

The card reader is disabled initially and is not supported by the default release of PIREX, the I/O executive that runs in the UC15.

The CPU model cannot be changed. While memory size can be changed, PIREX is configured for 16KB of memory.

The PDP-15/76 configuration requires the shared memory facility, which is presently implemented only for Windows and Linux.

## DR11 Parallel Interfaces (UCA, UCB)

The UC15 talks to the PDP-15’s DR15 interface over a pair of DR11-C interfaces called UCA and UCB. UCA implements these registers:

name			size		comments

CSR			16		control/status register

BUF			16		input buffer

APID			1		CSR<7>, API done

IE			1		CSR<6>, interrupt enable

POLL			10		polling interval

UCB implements these registers:

name			size		comments

CSR			16		control/status register

BUF			16		input buffer

NTCB			1		CSR<7>, new task control block

IE			1		CSR<6>, interrupt enable

UCA and UCB implement the SET/SHOW ADDRESS and SET/SHOW VECTOR commands, but if the address or vector of either interface is changed, PIREX will not run correctly.